AUWO Excavation Testbed

Single-Robot, Human-in-the-Loop, Safety-Interlocked Excavation

Architecture & Implementation Reference

CACDAR / DUL-derived design  ·  ROS 2 Jazzy  ·  RoArm-M2-S + UGV02  ·  Isaac Sim digital twin

# Contents

# 1. Overview

This document is the reference for the AUWO excavation testbed: what each part does, how the parts connect, and how to run them. It is written so a teammate or supervisor can understand both the architecture and the concrete ROS 2 implementation without reading the source first.

The system is a single robot — a RoArm-M2-S arm mounted on a UGV02 rover, with an OAK-D Pro camera — that performs an autonomous excavation cycle (dig, scoop, swing, dump) against a target. A human authorises each cycle, and an independent emergency stop can halt motion at any instant. The same control stack drives both a real arm and an Isaac Sim digital twin simultaneously.

The architecture borrows its vocabulary and layering from the CACDAR architecture (Mäkitalo et al., 2021) and the DUL (DOLCE+DnS Ultralite) ontology it builds on. Only the vocabulary and single-robot layering are adopted; the multi-robot cooperation machinery (peer models, cooperation messaging) is deliberately deferred until a second machine — a dumper — is introduced. The ontology is used as a design vocabulary now and would only become a runtime artifact (an OWL model with a reasoner) at the cooperation stage, when there is something to reason over.

## 1.1 Hardware and platform

| Component | Detail |
| --- | --- |
| Arm | RoArm-M2-S, ESP32 controller, /dev/ttyUSB0 @ 115200 |
| Rover | Waveshare UGV02 + Raspberry Pi 5 (edge compute) |
| Camera | OAK-D Pro (perception, not yet used for excavation) |
| Workstation | Ubuntu 24.04, NVIDIA RTX PRO 5000, ROS 2 Jazzy |
| Middleware | ROS 2 Jazzy, DDS, ROS_DOMAIN_ID=42 |
| Motion | MoveIt 2 + IKFast (position-only / translation3d IK) |
| Digital twin | Isaac Sim 5.1, mirrored arm following /roarm/joint_states |

Compute split. The Raspberry Pi (edge) runs the camera/perception front-end and the arm serial driver — the device-bound, high-bandwidth work. The workstation runs MoveIt, the planner, the world model, and Isaac. Only small semantic data (a target pose, a joint stream, a stop flag) crosses the DDS boundary between them; raw camera frames never leave the Pi.

# 2. Architecture

The control stack is organised as a 'cooperative brain' (using CACDAR's term) sitting above a substitutable world. Data flows in a closed loop: sensing produces facts, the knowledge manager holds them, the planner decides, a human confirms, the action executes, the world changes, and sensing observes the change. An independent emergency-stop path crosses this loop without passing through the planner.

## 2.1 Blocks and their CACDAR/DUL mapping

Each implemented piece maps to a named concept in the reference architecture. This is what makes the system legible against the paper and gives every future capability a named home.

| Our component | CACDAR / DUL concept | Status |
| --- | --- | --- |
| sim_target_bridge | Sense (perception front-end) | Stub stand-in |
| situational_context | Situational Context Model (DUL Situation) | Built |
| (none yet) | Environment Model (durable map) | Future |
| planner | Planner + Goal | Built |
| Confirm gate (in planner) | Human authorising Role | Built |
| excavation_server | Task: Excavate (DUL Task) + Action | Built |
| MoveIt / IKFast / compute_ik | Action implementation (platform) | Built |
| estop | Safety interlock (independent) | Built |
| Isaac Sim / real rover | World (3D Virtual / Real) | Built |
| Peer models, coop messaging | Cooperation layer | Deferred |

## 2.2 The Task / Action distinction

The central idea borrowed from the paper is the split between a Task and an Action. A Task is platform-independent: what to achieve, with a start condition, an end condition (an implicit low-level goal), and a resource estimate. An Action is the platform-specific implementation that achieves it. In our system, 'Excavate' is the Task — start condition 'a reachable dig target exists', end condition 'material moved to the dump point' — and the /excavate service running the dig-drag-scoop-lift-swing-dump cycle through MoveIt is the Action. Today the start/end conditions live implicitly inside the cycle (the reachability check, the cycle completing); the next refinement is to lift them into a declared Task contract.

## 2.3 Environment Model vs Situational Context

The knowledge manager has two stages that are easy to conflate. The Environment Model is the durable 'how the world is' representation — the dig region, surface height, dump zone, obstacles, the rover's place in a map. It changes slowly and is where perception writes. The Situational Context is the thin, current, derived snapshot the planner acts on — the specific dig target selected right now and whether it is reachable. The Situational Context is derived from the Environment Model plus live sensing.

Today there is no Environment Model and no excavation perception. A cone placed in Isaac stands in for both: sim_target_bridge reads the cone pose and writes the situational context directly. When real perception arrives it writes the Environment Model, a derive step produces the Situational Context, and nothing downstream of the situational context changes. That seam is intentional and is the highest-value future build.

## 2.4 The world as a substitutable layer

Isaac Sim is not a separate brain component; it is a world the architecture runs against, exactly as CACDAR frames its 3D Virtual World and Real World. The action layer outputs /roarm/joint_states, and whether that stream animates the Isaac arm, the real arm, or both at once is purely a matter of who subscribes. The digital twin therefore plugs in underneath the Task Runtime at the same seam the real robot does, and the two are interchangeable.

# 3. Data flow

The end-to-end path for one excavation cycle, from a target appearing to the arm moving:

- Isaac publishes the cone and robot transforms on /sim/target_tf (a TF message in the world frame).

- sim_target_bridge feeds those transforms into a private tf2 buffer, looks up base_link ← Cone (subtracting the robot's world pose), and publishes the target as a base_link point on /excavation/target (and /excavation/dump_target).

- situational_context consolidates the latest dig/dump targets, annotates each with a cheap reach-band check, and publishes a latched SituationalContext on /auwo/situational_context (also served by /get_situational_context).

- planner pulls the situational context, checks the goal is satisfiable (has dig target, reachable, e-stop clear), and prints a proposal. On /excavation/propose it arms a pending cycle; on /excavation/confirm it dispatches.

- excavation_server runs the cycle: it calls /compute_ik per waypoint and sends joint goals to MoveIt's /move_action, which drives /roarm/joint_states.

- Both the real arm (via roarm_driver) and the Isaac arm follow that joint stream — the world changes.

- At any instant, estop can publish a latched stop on /estop/stopped; the excavation server cancels its in-flight MoveIt goal and halts.

# 4. Node reference

All excavation nodes live in the auwo_task_planner package (ament_python). Custom messages and services live in auwo_interfaces (ament_cmake), which must be built first.

## 4.1 sim_target_bridge

Role: Sense stand-in. Converts Isaac cone poses into base_link target points.

- Subscribes: /sim/target_tf (tf2_msgs/TFMessage)

- Publishes: /excavation/target, /excavation/dump_target (geometry_msgs/PointStamped, base_link)

It uses a private tf2 buffer (not the global /tf) so it does not conflict with demo.launch.py's static world→base_link identity. It subtracts the robot's real world pose, so the target is correct wherever the robot sits. This requires base_link to be present among Isaac's published transforms — if Isaac's articulation is broken, base_link disappears and the bridge cannot transform.

## 4.2 situational_context

Role: The DUL Situational Context Model. Consolidates current targets and annotates reachability.

- Subscribes: /excavation/target, /excavation/dump_target

- Publishes (latched): /auwo/situational_context (auwo_interfaces/SituationalContext)

- Serves: /get_situational_context (auwo_interfaces/GetSituationalContext)

Message fields: header, has_dig, dig_target (Point, base_link), dig_reachable, has_dump, dump_target, dump_reachable. The reachability flags come from a cheap band check — radius and z bounds — that is deliberately approximate; the authoritative test is the action's IK. The band parameters were measured from the real arm (see section 5).

This node is read-only state: it never commands the arm or triggers a cycle. That separation is what lets the e-stop sit independently of it.

## 4.3 estop

Role: The single authority for the stop state — a safety interlock, not just a flag.

- Publishes (latched): /estop/stopped (std_msgs/Bool)

- Serves: /estop/trip, /estop/rearm (std_srvs/Trigger)

The stop state changes only through the trip / re-arm services, so the latching and deliberate-re-arm rules are enforced by construction — a stray topic publish cannot clear it. The state is broadcast on a latched topic so any consumer that starts after a trip comes up already knowing the system is stopped. A start_stopped parameter can arm the stop on launch.

## 4.4 excavation_server

Role: The Excavate Task and its Action. Runs the cycle and obeys the e-stop.

- Subscribes: /excavation/target, /excavation/dump_target, /estop/stopped

- Serves: /excavate (std_srvs/Trigger)

- Calls: /compute_ik, MoveIt /move_action

The cycle is APPROACH → BUCKET OPEN → DIG → DRAG → SCOOP → LIFT → SWING → DUMP DOWN → DUMP → RETURN. It is interruptible: it captures the in-flight MoveIt goal handle, and on an e-stop it calls cancel_goal_async so an in-progress motion aborts mid-waypoint rather than completing. The e-stop callback runs on a ReentrantCallbackGroup under a MultiThreadedExecutor, so it fires during a running cycle. A cancelled MoveIt goal returns code -7 (PREEMPTED) — the signature of a real mid-motion stop. The server also refuses to start a cycle while stopped.

Note on z_floor. A z_floor parameter clamps the minimum commanded z. It was originally set too high (0.04), which rewrote the cone's true depth and made the arm hover ~10 cm above the target. It is now set low enough to be a near-no-op so the real target depth reaches IK. The arm is mounted ~0.122 m up on the rover; base_link is the arm base, not the rover base.

## 4.5 planner

Role: Planner plus the human confirm gate, in one node.

- Subscribes: /estop/stopped

- Calls: /get_situational_context, /excavate

- Serves: /excavation/propose, /excavation/confirm (std_srvs/Trigger)

The confirm gate is part of the planner's dispatch path, not a separate node. /excavation/propose pulls the situational context, checks has_dig && dig_reachable (and, by parameter, dump_reachable) and that the e-stop is clear, prints the proposed dig→dump, and arms a pending cycle. /excavation/confirm dispatches /excavate. A trip clears any pending proposal. This two-call flow keeps the human's authorisation a deliberate, separate act — the same pattern as the e-stop's trip/re-arm.

## 4.6 reach_probe (offline tool)

Role: Measures the arm's true reachable workspace. Not part of the runtime stack.

It sweeps a grid of (x, y, z) points in base_link, calls /compute_ik on each, writes /tmp/reach.csv, and prints the empirical reach band. compute_ik is the authoritative reachability oracle — the same one the action uses — so the measured band reflects the real URDF, joint limits, and IKFast solver. The MoveIt workspace-analysis / Reuleaux tooling was considered and rejected as unmaintained on Jazzy.

# 5. The reach band

The situational context's reachability gate uses a measured band, not a guess. Running reach_probe over the workspace produced:

| Axis | Measured reachable range | Gate setting (with inset) |
| --- | --- | --- |
| radius √(x²+y²) | 0.014 … 0.528 m | 0.05 … 0.50 m |
| z | -0.170 … 0.400 m | -0.14 … 0.38 m |
| x | -0.100 … 0.500 m | (covered by radius) |

About 78% of the sampled box (9272 / 11907 points) was reachable. Two cautions: the reachable set is a dome, not a cylinder — at the extreme radius it is only reachable at certain z — so the band is a conservative inner approximation, which is fine for a planner pre-filter. And the gate band is inset a few centimetres from the measured extremes so it does not pass knife-edge, single-orientation solutions that may fail in a real trajectory. The authoritative check remains the action's IK during the cycle.

# 6. Frames and conventions

- Root frame: base_link is the arm base and the URDF root. The rover and camera are grafted on as fixed children. The arm base sits ~0.122 m above the ground on the rover.

- Targets: All excavation targets are expressed in base_link. The bridge does the world→base_link conversion.

- IK: Position-only (translation3d). 'Reachable' means the TCP can hit the point at some orientation IKFast returns — the same semantics the action commands with.

- Gripper units: RViz shows degrees (0–86°); MoveIt uses radians. Open ≈ 1.5 rad, grip ≈ 1.047 rad (60°).

- Mirroring: Both the real arm and the Isaac arm are open-loop followers of /roarm/joint_states. Isaac's joint-state graph must be a subscriber, never a publisher, or it fights the controller and floods empty joint states.

# 7. Build and run

## 7.1 Build

cd ~/Desktop/AUWO-testbed

colcon build --packages-select auwo_interfaces auwo_task_planner --symlink-install

source install/setup.bash

auwo_interfaces must build before auwo_task_planner. If the CLI reports an 'invalid message type', the terminal was sourced before the message was built — re-source.

## 7.2 Run the full stack

Each of the following is a separate process. The single launch file in section 8 starts the five auwo nodes together; MoveIt and Isaac remain separate (Isaac is a GUI; demo.launch.py is the MoveIt stack).

# 1. MoveIt + RViz + controllers

ros2 launch roarm_moveit demo.launch.py

# 2. Isaac Sim: open roarm_scene.usd, press Play

#    (ROS_DOMAIN_ID=42, base_link must be in the Transform Tree targetPrims)

# 3. The excavation stack (one launch file — see section 8)

ros2 launch auwo_task_planner excavation.launch.py

## 7.3 Drive a cycle (human-in-the-loop)

ros2 service call /excavation/propose std_srvs/srv/Trigger   # planner prints the plan, arms pending

ros2 service call /excavation/confirm std_srvs/srv/Trigger   # you authorise -> cycle runs

## 7.4 Emergency stop

ros2 service call /estop/trip  std_srvs/srv/Trigger   # halt now; cancels in-flight motion

ros2 service call /estop/rearm std_srvs/srv/Trigger   # deliberate re-arm to resume

# 8. The excavation launch file

This starts the five excavation nodes together. Place it at auwo_task_planner/launch/excavation.launch.py and add the launch directory to setup.py data_files (see section 8.2).

## 8.1 launch/excavation.launch.py

from launch import LaunchDescription

from launch_ros.actions import Node

def generate_launch_description():

pkg = 'auwo_task_planner'

common = dict(package=pkg, output='screen', emulate_tty=True)

return LaunchDescription([

Node(executable='sim_target_bridge',  name='sim_target_bridge',  **common),

Node(executable='situational_context', name='situational_context', **common),

Node(executable='estop',              name='estop',              **common),

Node(executable='excavation_server',  name='excavation_server',  **common),

Node(executable='planner',            name='planner',            **common),

])

## 8.2 setup.py — install the launch directory

Add an entry to data_files so the launch file is installed and discoverable:

import os

from glob import glob

# ... inside setup(...):

data_files=[

('share/ament_index/resource_index/packages',

['resource/' + package_name]),

('share/' + package_name, ['package.xml']),

(os.path.join('share', package_name, 'launch'),

glob('launch/*.launch.py')),

],

Then rebuild so the launch file installs:

cd ~/Desktop/AUWO-testbed

colcon build --packages-select auwo_task_planner --symlink-install

source install/setup.bash

ros2 launch auwo_task_planner excavation.launch.py

To pass parameters (for example a different reach band, or arming the e-stop on startup), add a parameters=[{...}] argument to the relevant Node entry.

# 9. Safety model and its limits

The human-in-the-loop and emergency-stop behaviours are two distinct mechanisms, by design:

- Confirm gate (precondition): the human authorises a cycle before it starts. It lives on the planner→action dispatch path.

- E-stop (interrupt): halts motion already underway, at any instant, independently of the planner. It cancels the in-flight MoveIt goal and holds position.

Honest limit. The e-stop is a best-effort software stop: it cancels goals and holds position, and because both arms follow the same joint stream, freezing that stream stops the sim and the real arm together. It is not a substitute for a hardware emergency stop that cuts motor power. On a real heavy machine, a hardware e-stop must sit underneath this software layer as the authoritative last line. The software layering is built so the hardware e-stop drops in beneath it without rearchitecting.

# 10. Deferred and future work

- Perception → Environment Model: replace the cone/bridge stand-in with real perception writing a durable Environment Model, plus a derive step that produces the situational context.

- Cooperation (DnS) layer: when a dumper is added, introduce the DUL Descriptions-and-Situations machinery — Descriptions defining Roles, classification of robots into Roles — and only then build an OWL ontology with a reasoner.

- Navigation: publish a live world→base_link (Isaac robot TF / odom), re-root the URDF on the mobile base, and add Nav2.

- Formal Task contract: lift the Excavate start/end conditions and resource estimate into a declared interface in auwo_interfaces, checked by the planner before dispatch.

- Hardware e-stop: a physical power-cutting stop beneath the software interlock, non-negotiable before any physical heavy-machine operation.

- Reachability refinement: optionally replace the box band with a nearest-neighbour check against the /tmp/reach.csv cloud to capture the dome shape.

Reference: N. Mäkitalo, S. Linkola, T. Laurinen, T. Männistö, “Towards Novel and Intentional Cooperation of Diverse Autonomous Robots: An Architectural Approach,” CASA@ECSA 2021.